Teltonika Hardware

| Attribute | FMC920 (Basic) | FMC130 (Professional — Primary) |
| --- | --- | --- |
| Category | Basic Tracker | Advanced Tracker |
| Primary Use | Light vehicles, trucks, support equipment | Heavy machinery: excavators, cranes, generators, loaders |
| 4G LTE | Cat 1 + 2G fallback | Cat 1 + 3G + 2G fallback |
| GNSS Systems | GPS, GLONASS, GALILEO, BEIDOU, SBAS, QZSS | GPS, GLONASS, GALILEO, BEIDOU, SBAS |
| Position Accuracy | < 2.5 m CEP | < 2.5 m CEP |
| GNSS Sensitivity | -165 dBm | -165 dBm |
| Internal Memory | 128MB (~422,400 records offline) | 128MB (~422,400 records offline) |
| Backup Battery | 170 mAh Li-Ion | 170 mAh Li-Ion |
| Bluetooth | 4.0 LE | 4.0 LE |
| Digital Inputs | 1 | 3 (incl. 1 Negative + 2 Impulse) |
| Analog Inputs | 1 | 2 |
| Digital Outputs | 1 | 3 (relay, immobilizer, buzzers) |
| CAN Bus | Via external OBD-II dongle only | Native LVCAN interface (direct CAN wiring) |
| Impulse Input (Fuel Flow) | No | Yes — for high-precision fuel flow meters |
| 1-Wire Support | No | Yes — temperature sensors + iButton operator ID |
| Negative Input | No | Yes — simplifies accessory wiring |
| Ingress Protection | IP54 (dust/splash proof) | IP41 (indoor/cabin mount) |
| Operating Temp | -40°C to +85°C | -20°C to +85°C |
| Weight | 54g | 55g |
| Dimensions | 79 x 43 x 12 mm | Small form factor, fits in tight spaces |
| Power Input | 10–30V DC | 10–30V DC |
| Overvoltage Protection | Up to 90V impulse (5ms) | Up to 90V impulse (5ms) |
| TRA / UAE Certified | Yes (CITC, TRA, CE/RED, E-Mark) | Yes (regional variants available) |
| Kasper Use | Support fleet, logistics trucks | ALL heavy equipment — PRIMARY CHOICE |

# FMC920 — Full Technical Specification

## Connectivity

- LTE Cat 1 with 2G fallback — future-proof for UAE's network evolution

- LTE FDD Bands: B1/B3/B7/B8/B20/B28 — covers all UAE carriers: du, Etisalat/e&

- Data rates: LTE up to 10 Mbps down / 5 Mbps up; GPRS up to 85.6 Kbps fallback

- GNSS: GPS + GLONASS + GALILEO + BEIDOU + SBAS — maximum satellite availability in desert terrain

- Bluetooth 4.0 LE for OBD-II dongles, temperature/humidity beacons, driver ID beacons

## Interface

- 1x Digital Input (DIN1) — ignition detection, door sensor, alarm button

- 1x Digital Output (DOUT1) — engine immobilizer relay, external LED

- 1x Analog Input (AIN1) — fuel level sensor, temperature probe, liquid level

- Micro-USB for local configuration via Teltonika Configurator software

## Built-in Scenarios

- Green Driving — detects harsh acceleration, braking, cornering via accelerometer

- Over-Speeding Detection — configurable speed threshold alerts

- Jamming Detection — alerts when GPS/GSM signal is actively blocked

- Crash Detection — accelerometer-based impact sensing

- Towing Detection — triggers when vehicle moves without ignition

- Unplug Detection — alerts when device is removed from power

- Auto Geofence — automatically sets a geofence around parked position

- Excessive Idling Detection — tracks engine-on without movement

- GNSS Fuel Counter — software-estimated fuel burn from movement data

- Trip monitoring — automatic trip start/end detection

## Power & Physical

- Power: 10–30V DC — compatible with all vehicle electrical systems

- Ultra Deep Sleep: < 2 mA — near-zero drain when asset parked for days

- IP54 rated — safe for engine bay, external chassis mounting

- Certifications: CE/RED, E-Mark, CITC, TRA, RCM, RoHS, REACH

# FMC130 — Full Technical Specification

## Connectivity

- 4G LTE Cat 1 + 3G (UMTS) + 2G fallback — three-layer network redundancy

- GNSS: GPS + GLONASS + GALILEO + BEIDOU — -165 dBm sensitivity

- Bluetooth 4.0 LE for wireless sensors and OBD-II dongles

- Internal flash: 128MB — stores up to 422,400 records when offline, dumps when reconnected

## Critical Interfaces for Heavy Equipment

- LVCAN interface — native CAN bus RX/TX lines for direct connection to machine ECU

- 3x Digital Inputs — ignition, door sensor, PTO (power take-off), hydraulic circuit status

- 1x Negative Input (DIN2) — simplified accessory wiring without dedicated ground lines

- 2x Impulse Inputs (DIN1, DIN2) — purpose-built for fuel flow meters (pulse counting = exact litres consumed)

- 2x Analog Inputs — 0–10V range, connects to fuel level probes, hydraulic pressure sensors

- 3x Digital Outputs — engine immobilizer relay, beacon light, buzzer

- 1-Wire temperature sensor — ambient, engine compartment, or cargo temperature

- 1-Wire iButton — operator ID authentication (machine will not start without registered key)

## Power & Environmental

- Power input: 10–30V DC (with overvoltage protection to 90V impulse)

- Operating temperature: -20°C to +85°C without battery — survives UAE summer equipment temps

- Ultra Deep Sleep: < 3 mA; Nominal with no load: < 33 mA

- Internal fuse: 3A, 125V — self-protective against wiring faults

- Weight: 55g — concealable inside equipment cab or control panel

# CAN Bus for Heavy Equipment

CAN Bus (Controller Area Network) is the internal computer network inside every modern vehicle or machine. Every sensor in a Caterpillar excavator, JCB loader, or Komatsu grader — the engine, fuel system, hydraulics, transmission — all communicate over the CAN bus. It is the machine's nervous system. The FMC130 can tap into this network and READ all of that data in real time. ► Without CAN access: GPS position only. ► With CAN access: position + engine hours + fuel level + RPM + fault codes + hydraulic status + more. This is the difference between basic tracking and genuine telematics intelligence.

## CAN Protocols

- J1939 — heavy equipment and truck standard. Used by Caterpillar, Komatsu, JCB, Volvo CE, XCMG, Liugong. This is what the FMC130 reads natively via its LVCAN interface.

- OBDII — light vehicle standard (cars, pickups, light trucks). FMC920 reads this via an external Bluetooth dongle.

- J1708/J1587 — older heavy equipment protocol (pre-2000 machines). Less common; some legacy Gulf fleet machines may use it.

- FMS (Fleet Management System) — standardised CAN gateway in European trucks. Accessible via 9-pin FMS connector under the dashboard.

## What CAN Data Gives You — AVL Parameter Examples

| AVL ID | Data Point | Business Use |
| --- | --- | --- |
| AVL ID 12 | Engine RPM | Detect operating vs. idle vs. off. Bill by engine-on hours. |
| AVL ID 16 | Total Mileage / Odometer | Virtual odometer — tracks exact machine distance |
| AVL ID 66 | External Voltage | Detect if machine power was cut — theft signal |
| AVL ID 67 | Battery Voltage | Machine battery health monitoring |
| AVL ID 68 | Fuel Level (%) | Real-time fuel level from tank sensor via CAN |
| AVL ID 72 | Engine Coolant Temperature | Predictive maintenance — overheat alerts |
| AVL ID 181 | Fuel Rate (L/hr) | Live fuel burn rate — catch abnormal consumption |
| AVL ID 182 | Total Fuel Used | Cumulative fuel used — essential for cost tracking |
| AVL ID 253 | Engine Total Hours | Maintenance scheduling (service every N hours) |
| AVL ID 10493 | DTC Fault Code (J1939 SPN) | Machine fault detection — push alert to operator |
| AVL ID 10494 | DTC Failure Mode (J1939 FMI) | Fault severity: active / pending / historical |
| AVL ID 525 | Engine Load % | Is the machine working hard or idling? Utilisation metric. |
| Custom | Hydraulic Status (Front) | Is the boom/arm/bucket actively operating? |
| Custom | Accelerator Pedal Position % | Operator driving behaviour on heavy machines |

# Recommended Accessories

| Accessory | Model / Type | Purpose | Applies To |
| --- | --- | --- | --- |
| CAN Adapter | Teltonika CAN-CONTROL / LVCAN adapter | Bridges FMC130 to vehicle CAN bus without splicing wires | FMC130 |
| iButton Reader | Teltonika iButton / Dallas DS1990A | Operator ID — machine won't start without registered key | FMC130 |
| BT OBD-II Dongle | Teltonika EYE OBD / Howen | Adds CAN data to FMC920 via Bluetooth, no wiring | FMC920 |
| Fuel Level Sensor | Technoton / LLS series (capacitive) | Ultrasonic fuel probe inserted into tank — high accuracy | FMC130 (AIN) |
| Fuel Flow Meter | Piusi / Aquametro series (impulse) | Inline flow meter — pulse counting via DIN impulse input | FMC130 (Impulse) |
| Temperature Sensor | Teltonika 1-Wire TSP01 | Engine bay or cargo temperature monitoring | FMC130 |
| EYE BLE Sensor | Teltonika EYE (BLE beacon) | Proximity / door open detection via Bluetooth | Both |
| External GNSS Antenna | Teltonika 003R-00253 | Improves GPS fix in metal-caged machinery cabs | FMC130 |
| Engine Immobilizer Relay | Standard 12V/24V relay (20A) | Wired to DOUT — remote engine disable via platform | FMC130 |
| AI Dashcam | Teltonika DashCam DCI200 | Driver behaviour video evidence with AI event tagging | FMC130 |